TABLE OF CONTENTS

normalize.filter/MMA_Normalize_Gain

normalize.filter/MMA_Normalize_Max

normalize.filter/MMA_Normalize_Min

normalize.filter/MMM_Pull

normalize.filter/MMM_Seek

normalize.filter/background

normalize.filter/MMA_Normalize_Gain

NAME

MMA_Normalize_Gain (V51) [.SGPQ], *double

FUNCTION

Returns absolute normalization gain for a stream. This attribute should

be queried after the first pass is done, so after connecting data source

to object's input. If queried before scan, it just returns 1.0.

Application may override the gain by setting it after the first pass is

done, it means after the input port is connected with source. It should

be noted however, that overdrive (by increasing gain above the value

computed in the first pass) is NOT saturated, so even small overdrive

will cause large and hard distortions.

NOTES

This attribute is passed via pointer to floating point double precision

number.

normalize.filter/MMA_Normalize_Max

NAME

MMA_Normalize_Max (V51) [..G.Q], *double

FUNCTION

Returns maximum sample value in an audio stream. This attribute should be

queried after the first pass is done, so after connecting data source to

object's input. Note that in case of integer streams it is the true

sample value, not scaled to [-1.0, +1.0] range.

NOTES

This attribute is returned via pointer to floating point double precision

number.

normalize.filter/MMA_Normalize_Min

NAME

MMA_Normalize_Min (V51) [..G.Q], *double

FUNCTION

Returns minimum sample value in an audio stream. This attribute should be

queried after the first pass is done, so after connecting data source to

object's input. Note that in case of integer streams it is the true

sample value, not scaled to [-1.0, +1.0] range.

NOTES

This attribute is returned via pointer to floating point double precision

number.

normalize.filter/MMM_Pull

NAME

MMM_Pull -- Writes normalized audio data to specified buffer. (V51)

SYNOPSIS

ULONG DoMethod(Object *obj, MMM_Pull, ULONG port, APTR buffer, ULONG

length);

FUNCTION

Pulls requested amount of audio data to destination buffer and then

multiplies it by normalization gain calculated during the first

normalization pass. As conversion is done in-place, no additional buffers

are allocated.

INPUTS

obj - object to perform method on.

port - number of port, data must be pulled from port 1 (output).

buffer - destination buffer for data. Must be aligned for AltiVec,

preferrably allocated with MediaAllocVec().

length - amount of data to be pulled in *bytes*. Must be an even multiply

of sample size. If not, it is rounded down.

RESULT

Number of bytes pulled, secondary error information via MMA_ErrorCode.

SEE ALSO

multimedia.class/MMA_ErrorCode

normalize.filter/MMM_Seek

NAME

MMM_Seek -- Moves stream pointer to a new position. (V51)

SYNOPSIS

ULONG DoMethod(Object *obj, MMM_Pull, ULONG port, ULONG type, UQUAD*

position);

FUNCTION

Perform seek in audio data stream. A normalize.filter object has no

persistent buffers, so it simply forwards the request from output to

input.

INPUTS

obj - object to perform method on.

port - number of port, seek must be done on port 1 (output).

buffer - type of seek, MMM_SEEK_TIME or MMM_SEEK_FRAMES.

position - a pointer to an UQUAD variable holding the new position. It is

in frames for frame seek and in microseconds for time seek.

RESULT

Boolean. FALSE in case of problems, secondary error information via

MMA_ErrorCode.

SEE ALSO

multimedia.class/MMA_ErrorCode

normalize.filter/background

DESCRIPTION

The filter purpose is to amplify an audio stream to maximize the

amplitude of signal without distortion. Normalization is done in two

passes. When the data source is connected to normalizer input, it scans

the whole stream to find peak values, then rewinds stream. Normalization

gain is calculated from peak values. When output of normalizer is

connected and MMM_Pull() is performed on the output, the signal is

multiplied by gain calculated.

Because of way of working, normalize.filter object requires that data

source is seekable. If application tries to connect a non-seekable data

source, MediaConnectTagList() will fail. It should be also noted that

application must be prepared for the first pass to take some amount of

time. Scanning pass is usually limited by mass storage media speed, but

stream decoder may also contribute. Becasuse of that, MediaConnect() call

connecting normalize.filter input to data source may take a few (or more)

seconds to complete.

In case when audio stream is multichannel (stereo or more), all channels

are normalized together, so balance between them is not changed.

NEW ATTRIBUTES

Attributes applicability:

I - may be set at creation time.

S - may be set on an existing object.

G - may be get from an object.

P - may be set for an object's port.

Q - may be queried from an object's port.

MMA_Normalize_Gain [.SGPQ], DOUBLE*

MMA_Normalize_Min  [..G.Q], DOUBLE*

MMA_Normalize_Max  [..G.Q], DOUBLE*

NEW METHODS

MMM_Pull(port, buffer, length)  (V51)

MMM_Seek(port, type, position)  (V51)

HISTORY

51.1 (15.09.2012)

- Initial release.