TABLE OF CONTENTS

pcix.library/PCIXAttemptBoard

pcix.library/PCIXCreateIntObjectTagList

pcix.library/PCIXDMAGetPhysical

pcix.library/PCIXDeleteIntObject

pcix.library/PCIXFindBoardTagList

pcix.library/PCIXGetBoardAttr

pcix.library/PCIXObtainBoard

pcix.library/PCIXReleaseBoard

pcix.library/PCIXSetBoardAttr

pcix.library/PCIXAttemptBoard

NAME

PCIXAttemptBoard - test and obtain boardobject

SYNOPSIS

Success=PCIXAttemptBoard(BoardObject)

D0                      A0

ULONG   PCIXAttemptBoard(void*);

FUNCTION

This function tries to obtain a board exclusive.

Every driver must do this before it can use the

hw.

The difference to PCIXObtainBoard is that it returns

with a failure if it couldn't obtain it.

INPUTS

BoardObject - a valid BoardObject

pcix.library/PCIXCreateIntObjectTagList

NAME

PCIXCreateIntObjectTagList - remove intobject

SYNOPSIS

IntObject PCIXCreateIntObject(BoardObject,Code,Data,MyTags)

A0          A1   A2   A3

void    *PCIXCreateIntObjectTagList(void*,void*,void*,struct TagItem*);

FUNCTION

This function creates an IntObject and adds it to

the board's interrupt chain.

INPUTS

BoardObject - the next BoardObject that matches the

given parameters or NULL for no more matches.

Code        - Interrupt Code. You can define the code type with

a tag. Useful for MorphOS.

Data        - Interrupt Data

MyTags      - Tag Stream to define additional parameters

o PCIXINTTAG_PRI, LONG

to define the local interrupt priority of the handler.

o PCIXINTTAG_MACHINE, ULONG

to define the handler's code type and calling convention.

MACHINE_M68k - 68k Mode, Default.

A1 = Data

A6 = SysBase

MACHINE_PPC  - PPC Mode (Only useful under MorphOS)

Arg1 = Data

Arg2 = SysBase

RESULTS

IntObject - the IntObject handle or NULL if the operation

fails.

pcix.library/PCIXDMAGetPhysical

NAME

PCIXDMAGetPhysical - Get a physical address for DMA use from

a given virtual address

SYNOPSIS

PCIXDMAGetPhysical( BoardObject, address)

A0       A1

void    *PCIXDMAGetPhysical(void*,void *);

FUNCTION

INPUTS

BoardObject - a valid BoardObject

address - a virtual address

SEE ALSO

BUGS

pcix.library/PCIXDeleteIntObject

NAME

PCIXDeleteIntObject - remove intobject

SYNOPSIS

PCIXDeleteIntObject(IntObject)

A0

void    PCIXDeleteIntObject(void*);

FUNCTION

This function removes the intobject from the

int server chain and if it's the last completely

disables the int server

INPUTS

IntObject - a valid IntObject

pcix.library/PCIXFindBoardTagList

NAME

PCIXFindBoardTagList - find a matching board entry

PCIXFindBoardTags - Varargs stub for PCIXFindBoardTaglist

SYNOPSIS

BoardObject = PCIXFindBoardTagList( OldBoardObject, MyTags)

D0                                 A0              A1

void*   PCIXFindBoardTagList(void*,struct TagItem*);

void*   PCIXFindBoardTags(void*,...);

FUNCTION

This function searches the list of existing Boards

structures in the system for the one that matches

the specified search attributes like Bus, Class,

Vendor and Device fields.

If the OldBoardObject is NULL the search is from the

start of the bus list.  If it is not null then

it searches from the board entry AFTER OldBoardObject.

There are specific wildcard ID codes defined for each

of the ID parameters.

INPUTS

OldBoardObject - a valid BoardObject, or NULL to start

from the beginning of the bus list.

MyTags         - the tag attributes to specify the search

filter attribtes.

RESULTS

BoardObject - the next BoardObject that matches the

given parameters or NULL for no more matches.

EXAMPLES

// to find all BoardObjects of the proper type

void *BoardObject = NULL;

struct TagItem MyTags[1];

MyTags[0].ti_Tag = TAG_END;

while (BoardObject = PCIXFindBoardTagList(BoardObject,

MyTags))

{

// do something with the returned BoardObject

// like PCIXGetBoardAttr()

}

SEE ALSO

BUGS

pcix.library/PCIXGetBoardAttr

NAME

PCIXGetBoardAttr - get informations about a board

SYNOPSIS

Info = PCIXGetBoardAttr( BoardObject, Attr)

D0                      A0           D0

ULONG   PCIXGetBoardAttr(void*,ULONG);

FUNCTION

This routine returns informations about the

board object dependent on the requested attribute

INPUTS

BoardObject - a valid BoardObject

Attr        - valid attributes listed below

o PCIXTAG_BUS (Read)

get the bus number where your board is located.

o PCIXTAG_BUSNAME (Read)

get the bus name where your board is located.

o PCIXTAG_IOBASE (Read)

get the IO Base or NULL. Useful to access non dynamic

ISA io addresses like serial, parallel, keyboard are

using. The size is always 1K when it exists.

o PCIXTAG_SLOT (Read)

get the slot number where your board is located.

o PCIXTAG_FUNCTION (Read)

get the function number of your boardobject.

A function number > 0 means that it's a multi

function board with more than 1 configheader.

There are also boards which aren't declared as

multifunction but with hidden configheaders.

Hidden configheaders's addressbases ARE NOT MAPPED

into the addressspace.

The Voodoo5 is such a board and you can search

for such configs with the PCIXFINDTAG_SHOWHIDDEN

tag for PCIFindBoardTagList()

o PCIXTAG_OWNER (Read)

get the owner task of the board.

Everybody which wants to use a board must use

PCIObtainBoard/PCIAttemptBoard.

o PCIXTAG_BASEADDRESS0 (Read)

o PCIXTAG_BASEADDRESS1 (Read)

o PCIXTAG_BASEADDRESS2 (Read)

o PCIXTAG_BASEADDRESS3 (Read)

o PCIXTAG_BASEADDRESS4 (Read)

o PCIXTAG_BASEADDRESS5 (Read)

get the configheader's baseaddress's address entry.

This is a virtual address is not guranteed to

be equal the physical.

o PCIXTAG_BASESIZE0 (Read)

o PCIXTAG_BASESIZE1 (Read)

o PCIXTAG_BASESIZE2 (Read)

o PCIXTAG_BASESIZE3 (Read)

o PCIXTAG_BASESIZE4 (Read)

o PCIXTAG_BASESIZE5 (Read)

get the configheader's baseaddress's size entry.

A size of 0 means it's not mapped.

o PCIXTAG_ROMADDRESS (Read)

get the board's copied romaddress.

This is not the real rom.

o PCIXTAG_ROMSIZE (Read)

get the board's copied romsize.

RESULTS

Information - depending on the requested attribute

SEE ALSO

BUGS

pcix.library/PCIXObtainBoard

NAME

PCIXObtainBoard - obtain boardobject

SYNOPSIS

PCIXObtainBoard(BoardObject)

A0

void    PCIXObtainBoard(void*);

FUNCTION

This function tries to obtain a board exclusive.

Every driver must do this before it can use the

hw.

Beware..if somebody else already owns the board

this function won't return until the ownership

is released.

INPUTS

BoardObject - a valid BoardObject

pcix.library/PCIXReleaseBoard

NAME

PCIXReleaseBoard - release boardobject

SYNOPSIS

PCIXReleaseBoard(BoardObject)

A0

void    PCIXReleaseBoard(void*);

FUNCTION

Release access to board. This function must be

called as soon as exclusive access to the hardware

is no longer needed. This allows other callers to

get access to the hw.

INPUTS

BoardObject - a valid BoardObject

pcix.library/PCIXSetBoardAttr

NAME

PCIXSetBoardAttr - set configheader entries

SYNOPSIS

Status = PCIXSetBoardAttr( BoardObject, Attr, Data)

D0                        A0           D0    D1

ULONG   PCIXSetBoardAttr(void*,ULONG,ULONG);

FUNCTION

This routine sets data into a board's configheader.

INPUTS

BoardObject - a valid BoardObject

Attr        - valid attributes listed below

o PCIXTAG_OWNER

Set a reference for a device driver

If a non-NULL reference has been set, it can only

be reset by setting it to NULL first

Data        - the Data which should be set.

RESULTS

Boolean - TRUE for success.

SEE ALSO

BUGS